VLAN Network Design

Configuration & Troubleshooting Documentation

Cisco Packet Tracer Lab

3 VLANs  |  13 Hosts  |  Inter-VLAN Routing  |  Layer 3 Switch

March 2026

# 1. Project Overview

This document describes the design, configuration, and troubleshooting of a VLAN-segmented network implemented in Cisco Packet Tracer. The network consists of three VLANs representing distinct organizational departments, interconnected via a Layer 3 switch with inter-VLAN routing enabled.

## 1.1 Objectives

- Segment network traffic into three isolated VLANs for security and manageability

- Assign appropriate subnets using Variable Length Subnet Masking (VLSM)

- Enable inter-VLAN routing using Switch Virtual Interfaces (SVIs) on a Layer 3 switch

- Verify connectivity both within and across VLANs

## 1.2 Equipment Used

- 1x Cisco Catalyst 3560 Layer 3 Switch

- 13x End-user PCs (5 IT, 5 HR, 3 Web-Servers)

- Copper Straight-Through cables

- Cisco Packet Tracer simulation environment

# 2. Network Design

## 2.1 VLAN Allocation

The network is divided into three VLANs. VLAN 10 and VLAN 20 each use a /27 subnet, providing 30 usable host addresses. VLAN 30 uses a /28 subnet providing 14 usable addresses for the 3 web servers.

| VLAN | Department | Hosts | Subnet | Gateway | Mask |
| --- | --- | --- | --- | --- | --- |
| 10 | IT | 5 | 192.168.10.0/27 | 192.168.10.1 | 255.255.255.224 |
| 20 | HR | 5 | 192.168.10.32/27 | 192.168.10.33 | 255.255.255.224 |
| 30 | Web-Servers | 3 | 192.168.10.64/28 | 192.168.10.65 | 255.255.255.240 |

## 2.2 IP Address Ranges

The table below details the full usable IP address range and broadcast address for each VLAN subnet.

| VLAN | Dept | Usable Range | Broadcast | Max Hosts |
| --- | --- | --- | --- | --- |
| 10 | IT | 192.168.10.2 - 192.168.10.30 | 192.168.10.31 | 30 |
| 20 | HR | 192.168.10.34 - 192.168.10.62 | 192.168.10.63 | 30 |
| 30 | Web-Svr | 192.168.10.66 - 192.168.10.78 | 192.168.10.79 | 14 |

## 2.3 Subnet Calculation Notes

VLAN 10 & 20 — /27 Subnetting

- Subnet mask: 255.255.255.224  |  Block size: 32  |  Usable hosts: 30

- VLAN 10: Network 192.168.10.0, Broadcast 192.168.10.31

- VLAN 20: Network 192.168.10.32, Broadcast 192.168.10.63

VLAN 30 — /28 Subnetting

- Subnet mask: 255.255.255.240  |  Block size: 16  |  Usable hosts: 14

- VLAN 30: Network 192.168.10.64, Broadcast 192.168.10.79

# 3. Switch Configuration

## 3.1 VLAN Creation

The following commands were entered in global configuration mode to create and name the three VLANs:

Switch> enable
Switch# configure terminal
Switch(config)# vlan 10
Switch(config-vlan)# name IT
Switch(config-vlan)# vlan 20
Switch(config-vlan)# name HR
Switch(config-vlan)# vlan 30
Switch(config-vlan)# name Web-Servers
Switch(config-vlan)# exit

## 3.2 Access Port Assignment

Switch ports were assigned to their respective VLANs in access mode:

! IT Department — Fa0/1 through Fa0/5
Switch(config)# interface range fa0/1-5
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 10
Switch(config-if-range)# exit

      
! HR Department — Fa0/6 through Fa0/10
Switch(config)# interface range fa0/6-10
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 20
Switch(config-if-range)# exit

      
! Web-Servers — Fa0/11 through Fa0/13
Switch(config)# interface range fa0/11-13
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 30
Switch(config-if-range)# exit

## 3.3 Inter-VLAN Routing (SVIs)

IP routing was enabled and Switch Virtual Interfaces were configured for each VLAN. Each SVI acts as the default gateway for hosts in that VLAN:

Switch(config)# ip routing

      
Switch(config)# interface vlan 10
Switch(config-if)# ip address 192.168.10.1 255.255.255.224
Switch(config-if)# no shutdown
Switch(config-if)# exit

      
Switch(config)# interface vlan 20
Switch(config-if)# ip address 192.168.10.33 255.255.255.224
Switch(config-if)# no shutdown
Switch(config-if)# exit

      
Switch(config)# interface vlan 30
Switch(config-if)# ip address 192.168.10.65 255.255.255.240
Switch(config-if)# no shutdown
Switch(config-if)# exit

      
Switch(config)# end
Switch# write memory

## 3.4 PC Configuration

Each PC was manually configured with a static IP, subnet mask, and default gateway via Desktop > IP Configuration in Packet Tracer:

- IT PCs: IPs 192.168.10.2-6  |  Mask 255.255.255.224  |  Gateway 192.168.10.1

- HR PCs: IPs 192.168.10.34-38  |  Mask 255.255.255.224  |  Gateway 192.168.10.33

- Web PCs: IPs 192.168.10.66-68  |  Mask 255.255.255.240  |  Gateway 192.168.10.65

# 4. Verification

## 4.1 VLAN Verification

The show vlan brief command confirmed all three VLANs were active with correct port assignments:

Switch# show vlan brief

      
VLAN  Name           Status    Ports
----  -------------- --------- --------------------------------
1     default        active    Fa0/14-24, Gig0/1-2
10    IT             active    Fa0/1, Fa0/2, Fa0/3, Fa0/4, Fa0/5
20    HR             active    Fa0/6, Fa0/7, Fa0/8, Fa0/9, Fa0/10
30    Web-Servers    active    Fa0/11, Fa0/12, Fa0/13

## 4.2 Routing Table

The show ip route command confirmed all three subnets were directly connected via their SVIs:

Switch# show ip route

      
192.168.10.0/24 is variably subnetted, 3 subnets, 2 masks
C  192.168.10.0/27   is directly connected, Vlan10
C  192.168.10.32/27  is directly connected, Vlan20
C  192.168.10.64/28  is directly connected, Vlan30

## 4.3 Connectivity Test Results

- ping 192.168.10.3  (same VLAN IT)         ->  SUCCESS  TTL=128

- ping 192.168.10.1  (own gateway VLAN 10)  ->  SUCCESS  TTL=255

- ping 192.168.10.33 (HR gateway VLAN 20)   ->  SUCCESS  TTL=255

- ping 192.168.10.65 (Web gateway VLAN 30)  ->  SUCCESS  TTL=255

- ping 192.168.10.36 (cross-VLAN HR PC)     ->  SUCCESS after ARP resolution

# 5. Troubleshooting Log

## 5.1 Issue Description

During initial testing, cross-VLAN pings to host IPs (e.g. 192.168.10.34 and 192.168.10.67) timed out, while pings to gateway IPs (SVIs) succeeded. Same-VLAN host pings also succeeded. This pattern was systematically investigated.

## 5.2 Diagnostic Steps and Findings

| Issue Observed | Investigated | Outcome |
| --- | --- | --- |
| Cross-VLAN ping timed out (.34, .67) | Wrong default gateway on destination PC | Gateway verified correct |
| Gateway pings succeeded (.33, .65) | SVIs up/up confirmed | Routing confirmed working |
| Same-VLAN ping succeeded (.3) | Suspected wrong subnet mask /24 vs /27 | Mask verified 255.255.255.224 |
| show vlan brief — all ports correct | VLAN assignments ruled out | No misconfiguration found |
| ipconfig on .36 verified correct | ARP cache / convergence delay | Resolved after ARP settled |

## 5.3 Root Cause

All device configurations were verified to be correct — VLANs, port assignments, subnet masks, and default gateways were all properly set. The connectivity failure was caused by ARP cache convergence delay in Packet Tracer. Upon the first ping attempts, the switch had not yet learned the MAC addresses of destination hosts. After ARP resolution completed, full inter-VLAN connectivity was established.

## 5.4 Key Diagnostic Commands

Switch# show vlan brief          ! Verify VLAN port assignments
Switch# show ip route            ! Verify routing table and connected subnets
Switch# show interface vlan 10   ! Verify SVI status (should be up/up)
PC> ipconfig /all                ! Verify PC IP address, mask, and gateway
PC> ping <destination-ip>        ! Test intra and inter-VLAN connectivity

# 6. Conclusion

The VLAN network was successfully designed and implemented in Cisco Packet Tracer. The following outcomes were achieved:

- Three VLANs (10, 20, 30) created and named for IT, HR, and Web-Servers

- All 13 PCs correctly assigned to their respective VLANs via access ports

- Inter-VLAN routing enabled and verified via SVIs on the Layer 3 switch

- Full connectivity confirmed both intra-VLAN and inter-VLAN

- Troubleshooting completed and root cause identified as ARP convergence delay

The network design demonstrates effective use of VLSM to minimize IP address waste while providing adequate address space for each department. The Layer 3 switch SVIs provide efficient inter-VLAN routing without requiring a separate router.